<question>What is PFD (Primary Flight Display)?

<variant> Primary flight data indicator

<variant> Engine running indicator

<variant>Hydraulic system status indicator

<question>What does the speed section on the PFD display?

<variant> Current airspeed

<variant> Aircraft weight

<variant>Air conditioning system pressure

<question>What color on the FMA indicates active mode?

<variant> Green

<variant> Red

<variant> Yellow

<question>Which instrument is the primary navigation indicator?

<variant> ND

<variant> ECAM

<variant> EICAS

<question>What does the altitude section on the PFD display?

<variant> Current altitude and vertical speed

<variant>Air temperature

<variant> Flaps condition

<question> Which cockpit element displays engine parameters on an Airbus?

<variant> ECAM

<variant> ND

<variant> PFD

<question>What parameter is displayed on the FMA?

<variant> Autopilot and autothrottle modes

<variant> ETA Forecast

<variant> Loading the luggage compartment

<question>What does the aircraft attitude indicator on the PFD show?

<variant> Pitch and roll

<variant> Wind speed

<variant>Fuel pressure

<question> Which scale on the PFD shows the speed limit?

<variant> Red grid (max speed)

<variant> Blue ribbon

<variant> Black Mark

<question>What does the purple triangle on the speed tape mean?

<variant> Set speed without auto traction

<variant> Minimum flight speed

<variant> Stall speed

<question>Where are the navigation waypoints displayed?

<variant> On ND

<variant> On ECAM

<variant> On FMA

<question>What is ECAM?

<variant> Electronic centralized aircraft monitor

<variant> Electronic fuel control module

<variant> Emergency power supply system

<question>Which indicator shows ILS information?

<variant> PFD

<variant> ECAM

<variant> FMS CDU

<question>What does A/THR mean on FMA?

<variant> Automatic traction

<variant> Automatic landing gear retraction

<variant> Automatic course stabilization

<question>What does the vertical speed display on the PFD?

<variant> Rate of climb/descent in feet per minute

<variant> Wind speed

<variant> Temperature gain rate

<question> Which system is responsible for automatic flight control along the route?

<variant> FMS

<variant> ECAM

<variant> EICAS

<question>What does ND show in PLAN mode?

<variant> The entire route is oriented north

<variant> Current speed

<variant> Engine parameters

<question>What does decision height mean?

<variant> The altitude at which the decision is made to continue landing

<variant> Minimum permissible flight altitude

<variant> Speed limit when dialing

<question>What does TCAS display on ND?

<variant> Other aircraft and distance to them

<variant> Cloudiness

<variant> Engine loading

<question>What is FCU used for?

<variant> To set autopilot parameters (speed, course, altitude)

<variant> To display navigation messages

<variant> For fuel control

<question2>What does the green circle on the PFD speed tape indicate?

<variant> Ideal recommended speed

<variant> Stall speed

<variant> Go-around speed

<question2>Why is the PFD considered the primary flight instrument?

<variant> It combines key data: speed, altitude, heading and orientation

<variant>He controls the autopilot

<variant> It replaces navigation charts

<question2>What does the FMA navigation section display?

<variant> Active Lateral Guidance Mode (HDG, NAV, LOC)

<variant>Air temperature

<variant> Remaining fuel

<question2>What is the purpose of different route colors on ND?

<variant> To differentiate between guided and unguided navigation modes

<variant> To separate turbulent zones

<variant> To display fuel levels

<question2> How does ECAM warn the crew about a malfunction?

<variant> Prints a message and highlights the corresponding system

<variant> Emits a constant beep

<variant> Blocks aircraft control

<question2>What does the blue writing on the FMA mean?

<variant> Specified but not activated mode

<variant>System error

<variant> Stall warning

<question2>What does the red hash mark on the speed tape mean?

<variant> Maximum permissible speed

<variant> Minimum speed

<variant> Recommended speed

<question2> In what mode does ND display ILS elements?

<variant> LS

<variant> ARC

<variant> PLAN

<question2> How does ECAM display engine parameters?

<variant> N1, N2, EGT and fuel consumption

<variant> Fan speed only

<variant> Interior temperature

<question2>What does G/S mean on FMA?

<variant> Active glide path guidance

<variant> Active course maintenance

<variant> Standby mode

<question2>What does the Flight Director show on the PFD?

<variant> Commands for proper aircraft control

<variant>Actual position of the aircraft

<variant> Wind direction only

<question2>What is the ND ARC mode used for?

<variant> Convenient display of surroundings and route

<variant> For landing approach only

<variant> For vertical navigation only

<question2>Why does TCAS display the altitude of other aircraft in hundreds of feet?

<variant> This is a standardized form of displaying relative altitude.

<variant> More convenient for pilots

<variant> Reduces CPU load

<question2>What does the blue triangle on the course feed mean?

<variant> Specified HDG direction

<variant>Actual exchange rate

<variant> Roll angle

<question2>What does the radio altimeter indicator show?

<variant> Altitude above ground below 2500 feet

<variant> Geometric height

<variant> Barometric altitude

<question3> Why is it unacceptable to exceed 30° of bank?

<variant>The protection systems are activated, keeping the aircraft within safe limits.

<variant> Autopilot is disengaged

<variant> The restriction applies only to landing

<question3> How does FMS use IRS and EICAS data?

<variant> Combines data from gyroscopes, accelerometers, and motor parameters to accurately calculate trajectory

<variant> Recalculates the course using GPS only

<variant> Uses auto-thrust data

<question3>Why does ND display the ETA forecast?

<variant> For precise planning of fuel, altitude and speed

<variant> To display crew change time

<variant> To calculate the glide path manually

<question3>Why is the combined mapping of ILS and FD important?

<variant> The pilot sees commands and deviations simultaneously, simplifying the approach

<variant> The load on the autopilot is reduced

<variant> Speeds up data output

<question3> How does ECAM prevent crew errors?

<variant> Provides a structured list of actions and highlights the failed system

<variant> Blocks control

<variant> Puts the aircraft into emergency mode

<question3> How does FMA display autopilot modes?

<variant> Separates modes into columns and displays their status in real time

<variant> Only blinks when an error occurs

<variant> Displays text notifications without structure

<question3> Why take into account the influence of wind on the course?

<variant> Wind drift compensation allows you to maintain the actual trajectory

<variant> For auto traction control

<variant> To calculate the load

<question3> How does the PFD provide autolanding?

<variant> Using ILS, radio altimeter, and autopilot data

<variant> Barometric altimeter only

<variant> GPS only

<question3>Why is a dangerous aircraft displayed in orange?

<variant> This indicates the TA/RA threat level according to international standards

<variant>This is the Airbus corporate identity

<variant> This reduces the load on the display.

<question3> Why does FMS compare data from two computers?

<variant> To detect discrepancies and prevent erroneous commands to the autopilot

<variant> To save fuel

<variant> To speed up ND output

<question>What is a number system?

<variant> A way of representing numbers using a set of symbols and rules

<variant> Data encryption method

<variant> Algorithm for sorting numbers

<question>What is the base of the decimal system?

<variant> 10

<variant> 8

<variant> 2

<question>What numbers are used in the binary system?

<variant> 0 and 1

<variant> 0–9

<variant> 1–8

<question>What is the base of the octal system?

<variant> 8

<variant> 16

<variant> 4

<question> How many digits does the hexadecimal system contain?

<variant> 16

<variant> 10

<variant> 8

<question>What letter corresponds to the number 10 in HEX?

<variant> A

<variant> C

<variant> F

<question> How to convert a binary number to decimal?

<variant>By multiplying the digits by powers of two and summing them

<variant> By dividing by two

<variant> By grouping by 4 digits

<question>What number system do computers use?

<variant> Binary

<variant> Decimal

<variant> Octal

<question>What does the entry (1011)₂ mean?

<variant> Binary number

<variant> Octal number

<variant> A number in hexadecimal notation

<question>What is used to represent hexadecimal digits above 9?

<variant> Letters A–F

<variant> Letters G–L

<variant> Letters M–R

<question> How to represent the number 8 in binary?

<variant> 1000

<variant> 111

<variant> 101

<question> Which number system is more compact for writing large numbers?

<variant> Hexadecimal

<variant> Binary

<variant> Decimal

<question> What does the index in a number notation show, for example (75)₁₀?

<variant> Base of the number system

<variant> Number of digits

<variant> Number sign

<question>What is the base of HEX?

<variant> 16

<variant> 8

<variant> 4

<question> How to convert a decimal number to binary?

<variant> Consecutively divide by 2 and write down the remainders

<variant> Multiply by 2

<variant> Group digits by 3

<question>What is a discharge?

<variant> The position of the digit in the number

<variant> Sign type

<variant> Fractional point

<question> Which system uses characters 0-7?

<variant> Octal

<variant> Binary

<variant> HEX

<question>What is the HEX value of the number 15?

<variant> F

<variant> E

<variant> C

<question>What does 2ⁿ mean in number systems?

<variant> The weight of the digit in the binary system

<variant> Discharge weight in HEX

<variant> Number of characters

<question>What are the characteristics of the positional number system?

<variant> The value of a digit depends on its position

<variant> Uses a fixed replacement table

<variant> The digits always have the same weight

<question2> How to convert (75)₁₀ to binary?

<variant> 1001011

<variant> 1010001

<variant> 111001

<question2> How to convert (1011)₂ to a decimal number?

<variant> 11

<variant> 9

<variant> 14

<question2>What is the minimum number of digits required to represent the number 15 in binary?

<variant> 4

<variant> 3

<variant> 5

<question2> How to convert binary number to octal?

<variant> Grouped by 3 bits on the right

<variant> Grouped by 4 bits from the left

<variant>Dividing by 8

<question2>What is the octal representation of the number (11100111)₂?

<variant> 347

<variant> 457

<variant> 721

<question2> How is conversion between binary and hexadecimal systems performed?

<variant> Grouped by 4 bits

<variant>Dividing by 3

<variant>Multiply by 16

<question2>What is the HEX representation of the number (1111 0001)₂?

<variant> F1

<variant> E3

<variant> B2

<question2> How to convert (632)₈ to decimal?

<variant> 410

<variant> 388

<variant> 512

<question2> How to convert decimal to binary?

<variant> Multiplying the fractional part by 2 and writing the whole parts

<variant>Dividing by 10

<variant>Multiply by 8

<question2> What is (F4C)₁₆ equal to in decimal form?

<variant> 3916

<variant> 1021

<variant> 2560

<question2> How to represent the number 8₁₀ in octal?

<variant> 10

<variant> 11

<variant> 12

<question2>Why is the hexadecimal system used?

<variant> It compactly displays binary data

<variant> It allows you to store more information

<variant> It replaces the binary system

<question2>What is the binary number (714)₈?

<variant> 111001100

<variant> 101001110

<variant> 100111011

<question2>What is the principle underlying positional systems?

<variant> Each digit has a weight equal to the base to the power of the position

<variant> Each digit has equal weight

<variant> Numbers do not change meaning in different positions

<question2> How to convert the number 70.68₁₀ to binary?

<variant> 1000110.101

<variant> 1100010.111

<variant> 1010101.011

<question3> Why is grouping done by 4 bits when converting a binary number to HEX?

<variant> 4 bits correspond to one hexadecimal digit

<variant> This is accepted historically

<variant> To reduce the number of errors

<question3> How does the base of the number system affect the length of the representation of a number?

<variant> The higher the base, the shorter the entry

<variant> The higher the base, the longer the entry

<variant> The base does not affect the record length

<question3> Why can converting fractions between number systems be inaccurate?

<variant> Some fractions have an infinite representation

<variant>Errors occur due to human rounding

<variant> The number systems are incompatible

<question3> Why can't (0.68)₁₀ be represented exactly in binary?

<variant> The fraction has an infinite binary representation

<variant>Not enough bits

<variant> This is a limitation of the decimal system.

<question3> How can I tell if the conversion was done correctly?

<variant> The inverse transformation must return the original number (including precision)

<variant> The number should decrease

<variant> The number should be rounded

<question3>What is the advantage of the octal system when working with binary data?

<variant> 3 bits are converted to 1 digit, making it easier to read

<variant> Requires less computation

<variant> Avoids rounding errors

<question3>Why is the 2nd system most efficient for digital electronics?

<variant> Devices easily implement two stable states

<variant> It is shorter than all systems

<variant> It's mathematically simpler

<question3> Why is it important to take into account the weights of the digits when performing transformations?

<variant> They define a numerical value

<variant> This speeds up calculations

<variant> This avoids rounding errors.

<question3>What is the difficulty in converting mixed fractions?

<variant> It is required to convert the integer and fractional parts separately

<variant> Requires a multi-bit processor

<variant> Relevance tables are needed

<question3>Why is HEX often used in programming?

<variant> It conforms to the binary data structure and is human-readable.

<variant> It is faster in calculations

<variant> It saves memory

<question>What is analog data?

<variant> Data changing continuously over time

<variant> Data consisting of bits

<variant> Data used only in computers

<question>What is digital data?

<variant> Discrete quantities represented by numbers

<variant>Continuous signals

<variant> Signals without quantization

<question>What is ADC used for?

<variant> To convert an analog signal to digital

<variant> To enhance the signal

<variant> For storing data

<question>What does a DAC do?

<variant> Converts a digital signal to analog

<variant> Measures temperature

<variant> Divides the signal into digits

<question>What is discretization?

<variant> Taking samples of an analog signal at equal time intervals

<variant> Data packaging

<variant>Signal smoothing

<question>What is quantization?

<variant> Approaching the signal value to the nearest level

<variant> Signal boost

<variant> Record the signal without changes

<question>What determines sampling frequency?

<variant> Number of samples per second

<variant> Number of quantization levels

<variant> Processor speed

<question>What is the minimum voltage required to change the ADC code?

<variant> LSB voltage

<variant> Supply voltage

<variant> Reset threshold

<question>What does EFSR mean?

<variant> Full ADC input voltage range

<variant> Sampling frequency

<variant> Quantization step

<question> How many levels does an 8-bit ADC have?

<variant> 256

<variant> 128

<variant> 512

<question>What are measuring transducers used for?

<variant> To generate an analog input signal

<variant> For storing digital data

<variant> For network transfer

<question>Which process comes first - quantization or discretization?

<variant> Sampling

<variant> Quantization

<variant> They are executed simultaneously

<question>What formula determines the quantization step?

<variant> Q = EFSR/(2^M - 1)

<variant> Q = M/EFSR

<variant> Q = 2·EFSR

<question>What is quantization error?

<variant> The difference between the original and quantized values

<variant> Processor error

<variant> Sampling frequency instability

<question>What does the comparator do in an ADC?

<variant> Compares the signal to the reference level

<variant> Boosts the signal

<variant> Multiplies the signal

<question>What is a tracking ADC?

<variant> An ADC that adjusts its output based on its input signal

<variant> The fastest ADC type

<variant> ADC for audio signals

<question>What does high resolution ADC show?

<variant> Large number of quantization levels

<variant> Higher sampling rate

<variant>Low power consumption

<question>What does an integrating ADC do?

<variant> Measures the duration of the integrated voltage discharge

<variant>Averages noise

<variant> Rotates the phase of the signal

<question>What system converts the signal back to analog?

<variant> DAC

<variant> ADC

<variant> Pressure sensor

<question>What happens at low sampling rates?

<variant> Loss of information and signal distortion (aliasing)

<variant> Decrease quantization accuracy

<variant> Increase PWM

<question2> In what case is discretization considered correct?

<variant> fs > 2 fmax

<variant> fs = fmax

<variant> fs < fmax

<question2>What happens when Kotelnikov's theorem is violated?

<variant> Frequency overlap (aliasing) appears

<variant> The signal level is decreasing

<variant> Quantization is lost

<question2> How does a successive approximation ADC work?

<variant> Progressively reduces the range of possible values

<variant> Compares all levels in parallel

<variant> Monitors signal changes in real time

<question2>What does an R/2R DAC do?

<variant> Generates an analog voltage corresponding to a binary code

<variant> Amplifies the digital signal

<variant> Corrects noise

<question2>Why is noise immunity important in digital systems?

<variant> Digital levels are easily restored in the presence of interference

<variant> Noise does not affect digital systems

<variant> Noise improves the accuracy of transformations

<question2> How does the ADC determine the output signal code?

<variant> By the level of the nearest quantum interval

<variant>By average signal strength

<variant> By signal frequency

<question2> Why does quantization lead to errors?

<variant> Actual values are rounded to the nearest level

<variant> The sampling frequency becomes lower

<variant> The signal intermittency disappears

<question2>What is the advantage of parallel ADC?

<variant>Very high speed

<variant>Low cost

<variant> High resolution

<question2> How does the number of comparators in a parallel ADC increase?

<variant> 2^n - 1

<variant> n * 2

<variant> n - 1

<question2>What is the typical error for an ADC with an unstable clock?

<variant> Aperture error

<variant> Quantum error

<variant> Rounding error

<question2> Why does a tracking ADC work effectively with slow signals?

<variant> They change smoothly, and the system has time to adapt

<variant> It has more quantization levels

<variant> It uses parallel comparators

<question2>What can reduce quantization noise in sigma-delta ADCs?

<variant> Oversampling and filtering

<variant> Using a large number of comparators

<variant> Voltage division

<question2> Why is a PWM-based DAC easy to implement?

<variant> You only need a current/voltage source and a filter

<variant> Requires many resistors

<variant> Requires an accurate DAC

<question2>What happens when the ADC resolution increases?

<variant> Increases accuracy and decreases quantization step

<variant>Increases sampling frequency

<variant> The number of levels is reduced

<question2>What is the advantage of a pipelined ADC?

<variant> Combines high speed and high resolution

<variant> The cheapest type of ADC

<variant> Has minimal error

<question3> Why does quantization inevitably lead to information loss?

<variant>An analog signal has infinite precision, but a digital signal does not.

<variant>Due to low sampling frequency

<variant>Due to the operation of comparators

<question3> Why is aliasing critical for control systems?

<variant> Distorted frequencies can result in incorrect control signals.

<variant> It causes a decrease in voltage

<variant> It reduces the number of quantization levels

<question3> Why are parallel ADCs rarely used for high bit depths?

<variant> Exponentially many comparators required

<variant>They are too slow

<variant>They do not support quantization

<question3> Why does quantization step affect noise?

<variant> The larger it is, the greater the deviation from the actual value

<variant> The error does not depend on the step

<variant> Step reduces harmonics

<question3>What is the difficulty in converting analog signals with high fmax?

<variant> High sampling rate required to avoid aliasing

<variant> Unable to perform quantization

<variant> Need a large DAC

<question3>Why does sigma-delta ADC provide high accuracy?

<variant> Noise is redistributed to high frequencies where it can be filtered out

<variant> It uses many comparators

<variant> It works without discretization

<question3>What limits the accuracy of an integrating ADC?

<variant> Instability of reference voltage and clock signal

<variant> Number of comparators

<variant> Number of quantization levels

<question3> Why does an R/2R DAC require precision resistors?

<variant>The slightest variation in resistance leads to errors in the analog output

<variant> To reduce the current

<variant> To increase the conversion frequency

<question3>What makes the comparator the key element in most ADCs?

<variant> It decides on the discharge threshold

<variant> It generates a clock signal

<variant>It filters out noise

<question3> Why is it important to know the quantization step for a correct inverse transform?

<variant> It defines the possible values of the signal

<variant> It specifies the sampling frequency

<variant> It only affects the noise level

<question>What is a data bus?

<variant> Common channel for transmitting information between devices

<variant> Data processing program

<variant> Microprocessor type

<question>What is the address bus used for?

<variant> To specify where to read or write data

<variant> For power transmission

<variant> To synchronize signals

<question>What does the control bus transmit?

<variant> Commands that control the operation of devices

<variant> Numeric data

<variant> Graphic information

<question>What does data bus width mean?

<variant> Number of bits transmitted simultaneously

<variant> Conductor length

<variant> Processor speed

<question>What does bus termination mean?

<variant> Impedance matching at line ends

<variant> Power off

<variant> Installing the driver

<question>What is a parallel bus?

<variant>Transmit multiple bits at once

<variant> Transfer data one bit at a time

<variant> Simultaneous operation of multiple processors

<question>What is a serial bus?

<variant> Transfer data one bit at a time

<variant> 8-bit data transfer

<variant> Data transfer in one direction only

<question>What does a bus controller do?

<variant> Controls data transfer between devices

<variant> Stores programs

<variant> Generates an image

<question> Example of a serial bus:

<variant> I2C

<variant> PCI

<variant> ISA

<question> Example of a parallel bus:

<variant> PCI

<variant> USB

<variant> SPI

<question>What is bus bandwidth?

<variant> The amount of data transferred per unit of time

<variant> Number of connectors

<variant> Tire length

<question>What does half-duplex mean?

<variant> Alternate data transfer in both directions

<variant> Transfer data simultaneously

<variant> One-way transfer only

<question>What does full-duplex mean?

<variant> Simultaneous two-way transmission

<variant> Read-only transfer

<variant> Write-only transfer

<question>What is bus arbitration?

<variant> The process of selecting a device to access the bus

<variant> Checking data for errors

<variant> Voltage conversion

<question>What is a bus master?

<variant> Device that controls the exchange

<variant> A device that only receives data

<variant> Data storage device

<question>What is a slave?

<variant> Device that executes the master's commands

<variant> Device that monitors the bus

<variant> Clock generating device

<question> What is the purpose of the clock signal on the bus?

<variant> Data transfer synchronization

<variant> Signal boost

<variant> Sending commands

<question>What is a star topology?

<variant> All devices are connected through a central node

<variant> All devices are connected in series

<variant> Devices operate without connections

<question>What type of bus does USB use?

<variant> Sequential

<variant> Parallel

<variant> Analog

<question>What does "interrupt" mean in the system bus?

<variant> Signal about the need for immediate event processing

<variant> Shutdown the device

<variant> Data loss

<question2> Why do parallel buses perform worse over long distances?

<variant> Due to interference, delays and line desynchronization

<variant> Require more power

<variant> Does not support full duplex

<question2>What is the advantage of serial buses?

<variant> High speed with a minimum number of lines

<variant>Simple increase in tire width

<variant> No interference

<question2>What makes the I2C protocol unique?

<variant> Allows you to connect multiple devices via two lines

<variant> Works only at high frequency

<variant> Transmits 16 bits at once

<question2> Why is SPI faster than I2C?

<variant> Uses separate lines for data and clock

<variant> Transmits 8 bits at once

<variant> Supports fewer devices

<question2> How does arbitration work in the CAN bus?

<variant>According to dominant and recessive levels on the line

<variant> Device by device

<variant> By clock time

<question2>What does CRC do in a CAN message?

<variant> Checks data integrity

<variant> Speeds up transmission

<variant> Increases throughput

<question2>Why does USB use differential pair?

<variant> To reduce noise and improve interference immunity

<variant> To increase the number of lines

<variant> For nutritional support

<question2>Why does the UART protocol require a precise frequency?

<variant> No clock line, synchronization between devices is internal

<variant>The data line has high impedance

<variant> Many slave devices are required

<question2>What does a handshake do when transmitting data?

<variant> Synchronizes the start, continuation, and end of a transfer

<variant> Increases nutrition

<variant> Removes data protection

<question2> Why was the parallel PCI bus replaced by PCI-Express?

<variant> PCI-E transfers data over high-speed serial lines

<variant> PCI-E is cheaper

<variant> PCI-E requires less power

<question2> How is reliability ensured in the CAN bus?

<variant> Dominance mechanism, error control, and retransmission

<variant> High signal voltage

<variant> No arbitration

<question2> How is addressing done in I2C?

<variant> Via 7- or 10-bit device addresses

<variant> Via MAC address

<variant> Via 3-bit code

<question2>Why isn't SPI used for long distances?

<variant> High sensitivity to interference and line length limitation

<variant> Low speed

<variant> Lack of slave devices

<question2>What is the purpose of message priority in CAN?

<variant> To ensure that critical data is transmitted first

<variant> For power distribution

<variant> To synchronize speed

<question2>What determines the speed of USB 3.0?

<variant> Operating frequency of the transmitting pair and coding efficiency

<variant> Number of devices

<variant> Data packet size

<question3> Why is CAN bus critical for automotive systems?

<variant> It provides reliable transmission in a highly interfering environment

<variant> She's the cheapest.

<variant> It transmits only analog signals

<question3>Why does PCI-E achieve high transfer speeds?

<variant> Uses dedicated lanes and serialization

<variant> Operates at low frequency

<variant> Does not use encoding

<question3>What is the difficulty of I2C arbitration with a large number of devices?

<variant>Increasing delays and collision probability

<variant> Reduce UART speed

<variant> Lack of power lines

<question3>Why does CAN use dominant and recessive levels?

<variant> For hardware conflict resolution and message priority

<variant> To reduce the current load

<variant> To transmit a beat

<question3> Why does USB-C support such high bandwidth?

<variant> Uses multiple high-frequency differential lines

<variant> Uses parallel data transfer

<variant> Transmits data in one direction only

<question3>What problems arise in parallel system buses?

<variant> Delay jumps, signal reflections, and crosstalk

<variant> Lack of slave devices

<variant>Weak error control

<question3> Why does the CAN bus withstand operation in harsh conditions?

<variant> Differential forwarding, hard error checking, and arbitration

<variant> High voltage

<variant>Low transmission frequency

<question3>Why is UART not suitable for high speeds?

<variant> Synchronization errors occur due to lack of clock line

<variant> Due to too many lines

<variant> Due to noise immunity

<question3>What limits the cable length in serial buses?

<variant> Signal attenuation and error growth

<variant> Address length

<variant> Number of devices

<question3> Why is PCI-Express architecture considered scalable?

<variant> You can increase the number of lines (x1, x4, x8, x16)

<variant> It is possible to increase the supply voltage

<variant> You can change the width of the parallel bus

<question>What is a logic element?

<variant> A device that performs simple logical operations

<variant> Calculation program

<variant> Memory for storing data

<question>What does the AND operation do?

<variant> Outputs 1 if both inputs are 1

<variant> Always returns 0

<variant> Returns 1 on any input

<question>What does the OR operation do?

<variant> Returns 1 if at least one input is 1

<variant> Returns 0 if both inputs are 1

<variant> Returns only 0

<question>What does the NOT operation do?

<variant> Inverts the input signal

<variant> Sums the signals

<variant> Returns a constant 1

<question>What is a logic diagram?

<variant> A set of connected logic elements

<variant> Memory type

<variant> Exchange protocol

<question>What does 1 mean in logic?

<variant>True

<variant> False

<variant> Error

<question>What does 0 mean in logic?

<variant> False

<variant>True

<variant> Uncertainty

<question>What is the name of the operation that produces 1 for different inputs?

<variant> XOR

<variant> AND

<variant> NAND

<question>What is the name of the element that performs the AND operation?

<variant> Conjunctor

<variant> Disjunctor

<variant> Inverter

<question>What is the name of the element that performs the OR operation?

<variant> Disjunctor

<variant> Inverter

<variant> Trigger

<question>What does the NAND operation do?

<variant> Inverts the result of AND

<variant> Repeats the result of OR

<variant> Calculates XOR

<question>What does the NOR operation do?

<variant> Inverts the result of OR

<variant> Adds bits

<variant> Multiplies bits

<question>What is a truth table?

<variant> A table of all possible inputs and corresponding outputs

<variant> Table of powers of two

<variant> List of processor commands

<question>What are basic logic elements?

<variant> AND, OR, NOT

<variant> USB, PCI, SPI

<variant> ADD, SUB, DIV

<question>What is a combinational circuit?

<variant> A circuit whose output depends only on the current inputs

<variant> Memory circuit

<variant> Clock signal circuit

<question>What does “Boolean algebra” mean?

<variant> Algebra that works with the logical values 0 and 1

<variant> Algebra that works with numbers

<variant> Matrix algebra

<question>What do the inputs of a logic gate do?

<variant> Receive signals that affect the output

<variant> Store data

<variant> Generates timing

<question>What does the output of a logic gate do?

<variant> Passes the result of a logical operation

<variant> Stores commands

<variant> Generates power

<question>What is the element that implements the NOR function?

<variant> NOR

<variant> NAND

<variant> XOR

<question>What is the element that implements the NAND function?

<variant> NAND

<variant> OR

<variant> XOR

<question2>What property makes NAND a universal element?

<variant> It allows you to build any logical functions

<variant>He works faster than everyone else.

<variant> It doesn't use power

<question2>What property makes NOR universal?

<variant> Any digital circuit can be built

<variant> It has fewer inputs

<variant> It requires less current

<question2> How is XOR output calculated?

<variant> Equals 1 if the inputs are different

<variant> Equals 1 if the inputs are equal

<variant> Always equal to 0

<question2>Why is minimization of logical functions necessary?

<variant> To reduce the number of elements and improve speed

<variant> To increase the number of wires

<variant> To complicate the scheme

<question2> Why are combinational circuits faster than sequential ones?

<variant> No state storage, response is generated immediately

<variant>Use more power

<variant> Always have one input

<question2>What is implication?

<variant> Logical operation “if A, then B”

<variant> Comparing numbers

<variant> Signal inversion

<question2> How is the XOR operation denoted?

<variant> ⊕

<variant> ×

<variant> ÷

<question2>What is a formula in perfect disjunctive normal form?

<variant> OR of AND sets of variables

<variant> AND of OR sets of variables

<variant> XOR of variables

<question2>What is a formula in perfect conjunctive normal form?

<variant> AND of OR sets of variables

<variant> OR of AND sets

<variant> XOR of variables

<question2>What does the trigger do?

<variant> Stores one bit of data

<variant> Calculates the sum

<variant> Boosts the signal

<question2>What is an SR flip-flop?

<variant> Memory element with Set and Reset inputs

<variant> Pulse counter

<variant> Frequency divider

<question2> Which element is used to add binary numbers?

<variant> Half adder

<variant> Trigger

<variant> Inverter

<question2>What does a full adder do?

<variant> Adds 2 bits and carries

<variant> Adds 3 bytes

<variant> Adds numbers only without carrying

<question2>What is called propagation delay?

<variant> Time between input change and output change

<variant> USB transfer time

<variant> Processor latency

<question2> Why is it important to consider logic gate delays?

<variant> They affect the correct operation of the circuit

<variant>They reduce energy consumption

<variant>They speed up operations

<question3>Why does delay accumulation occur with a large number of logic levels?

<variant> Each element adds its own delay, and the total time increases

<variant> Due to increased energy consumption

<variant>Due to reduced throughput

<question3> Why does minimizing logic functions reduce power consumption?

<variant> Fewer elements → fewer switching operations → lower energy consumption

<variant> The scheme works faster

<variant>The supply voltage is decreasing

<question3>Why does cascading adders limit speed?

<variant> The transfer goes through a chain of elements

<variant> Too many wires

<variant> Multiple levels of OR

<question3>Why is XOR often used in cryptography?

<variant> It provides a simple but robust two-way data modification operation

<variant>He is the fastest

<variant> It does not require power

<question3> Why are NOR gates more difficult to analyze?

<variant>They use double inversion and require careful function construction.

<variant>They consume more current

<variant>They do not support high-level inputs

<question3> Why are race conditions taken into account in design?

<variant> Due to different delays, output uncertainty arises

<variant>Due to power supply instability

<variant> Due to other logical elements

<question3> Why do combinational circuits require strict optimization?

<variant> Extra levels reduce speed and increase crystal area

<variant> Schemes are becoming less convenient

<variant>The voltage decreases

<question3>Why are flip-flops critical for sequential circuits?

<variant>They store state, ensuring operation on a clock cycle

<variant>They reduce the number of operations

<variant>They operate without power

<question3>What limits the maximum frequency of digital circuits?

<variant> Total delays of logic elements

<variant> Number of variables in the function

<variant> Number of triggers

<question3> Why does logic optimization affect the cost of a device?

<variant> Fewer elements → smaller area → lower production costs

<variant> Fewer wires are used

<variant> Memory size is decreasing